# Symbian OS 6.1 Comms Setup Guide

Last revised: 16th June 2001, Version 004

Copyright © Symbian Ltd. 2001. All Rights Reserved.

Released via the Symbian Developer Network – http://www.symbiandevnet.com/

# Introduction

# With V6.1 of the Symbian OS we are providing some new tools and guidelines to assist developers in comms testing/development with the Symbian Quartz 6.1 Software Development Kit (SDK). This document will explain how to setup the follow communications options:

- Windows NT RAS

- Dial-up via an Infrared ‘pod’ attached to your PC to an IrDA-enabled mobile phone

- Dial-up via a Bluetooth ‘board’ attached to your PC to a Bluetooth-enabled mobile phone

# Common Steps

# The following should be carried out on the Quartz emulator prior to attempting any specific comms setup. These steps apply to all of the supported communications options outlined in this document.

- Rename the file <Install Folder>\epoc32\release\wins\udeb\watcher.dll to something else such as watcher.not_used

- Rename the file <Install Folder>\epoc32\release\wins\udeb\z\system\libs\watcher.dll to something else such as watcher.not_used

- Install the Phone On/Off tool supplied with the Quartz 6.1 SDK. Details can be found in the SDK Release Notes

- Install the tool provided with this document, CommConfig, onto your SDK folder using the path names stored inside the ZIP file. This tool provides automated-configuration of the Symbian OS Communications Database (COMMDB) which is stored at <Install Folder>\epoc32\wins\c\system\data\Cdbv2.dat

# Infrared (IrDA) Communication Settings

To configure the emulator to use an IrDA connection, do the following:

- Close down any application which uses/polls the COM ports on your PC (e.g. PsiWin, Symbian Connect, etc.). If you have previously configured NT RAS on your PC with the emulator (see section 5 in this document) then also ensure that the RAS service is stopped from inside the Remote Access Admin tool supplied with Windows NT.

- Connect your IR pod to COM2 on your PC. If you have a laptop with only a single COM port then connect the IR pod to this port and then edit the file <Install Folder>\epoc32\wins\c\system\data\irda_wins.esk to change the line:

irPhysicalComPort= 1

to:

irPhysicalComPort= 0

- From a command prompt, change to the SDK folder <Install Folder>\epoc32\release\wins\udeb\ and issue the command:

CommConfig.exe –Mconsole --

to launch the CommConfig tool. Opt to overwrite any existing CommDB file.

- Select option (b) to create the configuration for an IrDA Mobile Phone

- Launch the Quartz WINS UDEB emulator

- If this is the first time you are setting up the IrDA connection after step 5, follow steps 7 and 8. Otherwise skip ahead to Step 9.

- Go to Control Panel and select Link to desktop. Make sure that the link is set to Cable and not Infrared.

- Also in Control Panel, select Internet accounts so you can enter some Internet Service Provider (ISP) information. Edit the ‘Dummy ISP with IrDA Phone’ and set the fields of the dialog to an ISP of your choice. Confirm the dialog box with the Done button.

- From the Quartz AppPicker, run the Phone On/Off application. Select the button marked Phone On/Off. You should see a message telling you the ‘phone’ being emulated by Quartz has been switched off. This is essential because otherwise the ports on your PC will be used by this application and will not be available for communications. On a real device, this would not be an issue – it is only a problem on WINS when trying to ‘force’ the emulator into ‘two-box’ mode from its default ‘one-box’ setup.

- The settings used by the CommConfig EXE are ones designed to work with most IrDA phones (they equate to the ‘Infrared mobile phone’ modem as seen in the ER5 Dialling Control Panel). Depending on which phone you are using, you may need to manually tweak some settings (e.g. Init string, handshaking, etc.). To do this, use a command prompt to change to the SDK folder <Install Folder>\epoc32\release\wins\udeb\ and issue the command:

t_editdb.exe –Mconsole --

to launch the CommsDB Editor tool. You can then alter individual Modem settings.

- That’s it! Position your phone near the IR pod attached to your PC, enable the IR port on your phone if needed and try running the Web application to connect to a page!

# Bluetooth Communication Settings

To configure the emulator to use a Bluetooth connection, do the following:

- Close down any application which uses/polls the COM ports on your PC (e.g. PsiWin, Symbian Connect, etc.). If you have previously configured NT RAS on your PC with the emulator (see item 5 in this document) then also ensure that the RAS service is stopped from inside the Remote Access Admin tool supplied with Windows NT.

- Connect your Bluetooth board COM1 on your PC

- Edit the file <Install Folder>\epoc32\wins\c\system\data\CommConfig.ini to update the section marked [DEFAULT_BT_DEVICE] with the unique ID of the mobile phone you are trying to connect to. This should be listed on the phone/add-on Bluetooth hardware somewhere.

- From a command prompt, change to the SDK folder <Install Folder>\epoc32\release\wins\udeb\ and issue the command:

CommConfig.exe –Mconsole --

to launch the CommConfig tool. Opt to overwrite any existing CommDB file.

- Select option (c) to create the configuration for a Bluetooth Mobile Phone. In addition to setting up the Comms Database, this will automatically attempt to read the CommConfig.ini file mentioned above to obtain (and set in the Comms Database) the address of your default Bluetooth device.

- Launch the Quartz WINS UDEB emulator

- If this is the first time you are setting up the Bluetooth connection after step 6, follow steps 8 and 9. Otherwise skip ahead to Step 10.

- Go to Control Panel and select Link to desktop. Make sure that the link is set to Cable or Infrared and not Bluetooth.

- Also in Control Panel, select Internet accounts so you can enter some Internet Service Provider (ISP) information. Edit the ‘Dummy ISP with Bluetooth Phone’ and set the fields of the dialog to an ISP of your choice. Confirm the dialog box with the Done button.

- From the Quartz AppPicker, run the Phone On/Off application. Select the button marked Phone On/Off. You should see a message telling you the ‘phone’ being emulated by Quartz has been switched off. This is essential because otherwise the ports on your PC will be used by this application and will not be available for communications. On a real device, this would not be an issue – it is only a problem on WINS when trying to ‘force’ the emulator into ‘two-box’ mode from its default ‘one-box’ setup.

- The settings used by the CommConfig EXE are ones designed to work with the Ericsson T28 World phone and Bluetooth dongle. Depending on which Bluetooth hardware you are using, you may need to manually tweak some settings (e.g. Init string, handshaking, etc.). To do this, use a command prompt to change to the SDK folder <Install Folder>\epoc32\release\wins\udeb\ and issue the command:

t_editdb.exe –Mconsole --

to launch the CommsDB Editor tool. You can then alter individual Modem settings.

- That’s it! Enable Bluetooth on your phone if you need to and then try running the Web application to connect to a page!

# NT RAS Communication Settings

The NT RAS software enables the PC to act as a gateway to the local area network and the Internet. The normal use of RAS is to enable remote users to connect to the network via a modem connected to the PC. However, it can also be used as a means of enabling Symbian OS emulators to obtain network access.

On PCs with two or more serial ports, it is recommended that RAS be configured to use the second port (COM2). This enables both the emulator and the RAS server to run on the same machine. This requires the use of a NULL modem cable to be connected between your two COM ports.

There are two sets of steps for configuring the Quartz emulator to use an NT RAS connection – some under Windows NT, some under Quartz:

# NT RAS – Windows NT Setup

- Network Dialog
· From the Windows control panel, select the Network icon
· Choose the Services tab
· This will display a list of the services which are currently installed
· If Remote Access Service is not already installed, press the Add button and go to step 2
· If Remote Access Service is present, highlight it and then press the Properties button. Now skip step 2 and go to step 3

- Select Network Service Dialog
· Highlight Remote Access Service and press OK
· This will asked for a location for the files - which is generally correct. Press OK
· After it has been installed, you should be asked if you wish to install a modem. Press Yes
· Now skip steps 3 & 4 and go to step 5

- Remote Access Setup Dialog
· Press the Add button

- Add Ras Device Dialog
· Press the Install Modem button

- Install New Modem Dialog
· Select the "Don't detect my modem" check box and then press Next>
· Under models, select Dial-Up Networking Serial Cable between 2 PCs (the first item in the list), then press Next>
· Select the port to wish to use. On a PC with two serial ports, this will normally be COM2, then press Next>
· Press Finish

- Remote Access Setup Dialog
· Select the COM port you have just installed then press Configure

- Configure Port Usage Dialog
· Select Receive call only, then press OK
· If RAS has been set up on 2 serial ports only one port can be configured to receive calls.

- Remote Access Setup Dialog
· Select Network…

- Network configuration Dialog
· Under Server Settings group, make sure only TCP/IP is selected
· Under Encryption Settings, select "Allow any authentication including clear text"
· Press OK

- Remote Access Setup Dialog
· Press the Continue Button

- Network Dialog
· Press the Close button

- Reboot Dialog
· This will ask you if you want to reboot, but press NO for now

- Modem Set Up
· From the Windows control panel, select the Modems icon
· Chose the Port which you have just configure for NT RAS and press Properties
· In the General tab, select a Maximum speed of 115200
· In the Connections tab, choose Data Bits: 8, Parity: None, Stop Bits: 1
· In the Connections tab, press the Advanced button
· Select Use flow control with the option of Hardware (RTS/CTS)
· Close and save all modem related dialogs.

- User Manager
· From the Windows start menu Choose Administrative Tools (Common) => User Manager
· In the user manager screen, select User => New User from the menus
· Enter the Username of ras
· Enter a Password of ras
· De-select the option User Must Change Password at Next Login
· Select Password never expires - This should be the only checked item.
· Press OK on the new user dialog
· Exit from the User Manager screen

- Now reboot the machine

- Remote Access Admin
· From the Windows start menu, Choose Administrative Tools (Common) => Remote Access Admin
· From Remote Access Admin screen, select Users => Permissions from the menus
· Highlight the user ras and then select Grant dialin permission to user.
· Press OK
· Again from the application's menus, choose Server => Start Remote Access Service
· This will display a dialog with the name of the Server (your machine name), press OK

- NT RAS has now been configured on your machine!

# NT RAS – Quartz Setup

- Close down any application which uses/polls the COM ports on your PC (e.g. PsiWin, Symbian Connect, etc.)

- Ensure the NULL modem cable is connected on your PC and that the RAS service is enabled and running (see section 5.1).

- From a command prompt, change to the SDK folder <Install Folder>\epoc32\release\wins\udeb\ and issue the command:

CommConfig.exe –Mconsole --

to launch the CommConfig tool. Opt to overwrite any existing CommDB file.

- Select option (a) to create the configuration for NT RAS.

- Launch the Quartz WINS UDEB emulator

- If this is the first time you are setting up the NT RAS connection after step 5, follow steps 7 and 8. Otherwise skip ahead to Step 9.

- Go to Control Panel and select Link to desktop. Make sure that the link is set to Infrared and not Cable.

- Also in Control Panel, select Internet accounts if you wish to alter the RAS username/password. Make any required changes and then confirm the dialog box with the Done button.

- From the Quartz AppPicker, run the Phone On/Off application. Select the button marked Phone On/Off. You should see a message telling you the ‘phone’ being emulated by Quartz has been switched off. This is essential because otherwise the ports on your PC will be used by this application and will not be available for communications. On a real device, this would not be an issue – it is only a problem on WINS when trying to ‘force’ the emulator into ‘two-box’ mode from its default ‘one-box’ setup.

- That’s it! Try running the Web application to connect to a page!